Mini DVR User Menu

Quick View

1、REC/ Pause/Power          5、Power Switch           9、MIC

2、Red indicator               6、T-Flash(MicroSD) slot   10、Mode Switch (Photo/ Video)

3、Green indicator             7、USB port

4、Blue indicator              8、Camera

Operation Instruction

1. Description of indicators:

a. Green indicator: the power indicator

b. Blue indicator: “vide mode”

c. Red indicator: “photo and voice record mode”.

2. Description of Mode Switch:

* when it is moved to the photo direction as marked, DVR will goes to take picture

* when it is moved to the video direction as marked, DVR will goes to video record and voice record  state

3. Startup Video Record or Take pictures:

*Video Record: when the “Power switch” was set to ON , “mode switch”set to video, press “REC”key 3seconds around ,the green indicators will turn on and the DVR goes to the initialization state. After the completion of initialization, the Blue indicator will turn on and you can start to recording video now (press “REC” key again). the Blue indicator will slow flash when the DVR is recording. Short press the button in the recording state to stop recording and save the video file.

*Take Picture: when the “Power switch” was set to ON , “mode switch”set to Photo ,press “REC”key 3seconds around, the green indicators are on and the DVR goes to the initialization state. After the completion of initialization, the Red indicator will trun on and you can start to take pictures now. the Red indicator will flash one time when you press “REC” key every time.

*webcam function: mode key Switch to “Photo”side, Put in USB port, and install driver when your system found new hardware. (all driver files in CD disk)

*Voice Record: in video side, keep play key 3 seconds around, DVR will go in voice record state. and Red Led will slow flash.

Note:

a.If all indicatores were flashing and turn off automatically ,it mean the memory is fully, please erase some file or change new memory card.

b. If the battery is low during recording, the DVR will save the file before turn off. And it will turn off automatically,please charge for use of next time.

4. Turn off: Pause it the first, and switch “power switch” to off.

5.Charging:

a. charge DVR by PC, Plug in the USB Cable in STANDBY state and connect it to the computer’s USB Interface.Blue indicator will flash regularly if the battery is low—meaning that the unit is in the general charging state, please click on the USB Disc icon in the Task List at the lower right corner to unload the disc (do not disconnect the USB Cable) and then long press the button to flash the blue indicator regularly meaning the charging in the ON state. When the battery is charged fully, the Blue indicator flashes permanently.

b.Charge DVR by charger, Plug in thecharger in STANDBY state or power on state.Blue indicator will flash regularly if the battery is low—meaning that the unit is in the general charging state, it will stop flash if the batty is fully.

Note: the DVR is provided with the built-in hi-capacity lithium battery. The battery has no memory effect and can be charged at anytime, but it has to be charged fully after having been used up for the first five times of charging so as to activate the maximum capacity of the battery and obtain the standard operating time. Users are recommended to charge it in OFF state (the blue indicator flashes then).

6. Time setting: the DVR can display the recording time on the video file in the format of Year.month.day hour: min: second. Such time can be set up as below:

a. Create a notepad file with the extension name “time.txt” and enter a time as shown in the format 2009.07.15 16:10:28. Such time is your current local time. Please check the text formatting carefully to ensure a correct format. Or you can modify directly in the CD-contained example file and then save and apply.

b. Connect the DVR with a computer and copy “time.txt” into the root directory of USB Disc after the appearance of USB Disc on the screen.

c. Disconnect the hardware，turn on and turn off one time，new time will record.

7. Rest DVRsome time, the DVR will stop reacting occasionally due to misoperation or other reasons. If so, please turn off the power supply directly and turn on it again.

Specification

* Video : 640*480 30FPS AVI format

Note: (Recommend use Kmplayer Play it, free software in CD disc, also you can download it from website: http://www.kmplayer.com/forums/forumdisplay.php?f=5 ).

* Photo : 1600 X 1200  JPG format

* Memory Support: T-Flash card (Micro SD) Max: 32GB

*Support system: Windows98SE/ME/2000XP /LINUX2.4 above and automatically distinguish operating system,for win98 system, please install win98 driver files in CD disk

* Battery type: High Capacity Polymer Li-ion Battery.

* Charge Voltage:DC-5V.

* USB type: USB standard port, need no drive or circumscribed electric power source neither.

Precautions

* Applicable occasions: you have to abide by the applicable national laws and regulations and this product shall not be applied to other illegal purposes or you shall be responsible for the consequence thereof.

* Software upgrading: Please inquire the local distributor for information on the latest software version and upgrading methods.

* Operating temperature: please use it at natural temperature and do not use at a temperature that human body cannot withstand.

* Operating humidity: Please use it at the ambient humidity fit for human life and do not place it in a humid operating condition. This product is not waterproof.

* Operating illuminance: please use it under a condition with sufficient light source and do not aim the DVR directly to the sun or other strong light source or the optical parts may be damaged.

* Cleaning: Do not use it under a dusty condition so as to keep its lens and other parts from dust or the effect of video will be affected. The lens may be cleaned with lens wiping paper or glass towel.

* Others: this product is precision electronic product and do not make it subject to any impact or vibration; do not use in strong magnetic field or electric field.

- 总览

1、录制/开机按钮         5、电源开关               9、麦克风

2、蓝色指示灯            6、T-Flash(McroSD)卡      10、模式切换 (照相录音/ 录像)

3、橙色指示灯            7、USB口

4、电源指示灯            8、摄像头

二、快速使用

1.录像操作

把模式开关拨向“video”,电源开关拨向“ON”,按“录制”按钮3秒左右，电源指示灯（绿色）将亮起，当蓝色灯亮起的时候再次短按“录制”按钮，蓝色灯将会慢速闪烁，表明正在录制中，录制过程中如果按下录制按钮将转为暂停，蓝色灯停止闪烁

**在录像暂停状态把模式开关拨向“Photo”即可直接切换到拍照模式

**在录像模式下，如果需要转换到录音状态请长按3秒可进入录音模式，录音过程中红灯慢闪，再次按 REC 键可退出录音状态，返回待机模式.

**如果在录制过程中出现3个指示灯同时闪烁几次后自动关机为内存已经用完,请更换内存卡或删除无用档以更新空间

2.拍照操作,录音操作

把模式开关拨向“Photo”,电源开关拨向“ON”, 按下“录制”开关3秒左右，电源指示灯（绿色）将亮起，当橙色灯亮起的时候再次按下“录制”按钮，红色灯将会快速闪烁一次，表明拍照过程已结束，长按3秒即可关机.

**在拍照暂停状态把模式开关拨向Video即可直接切换到录像模式

3.网络摄像头功能：模式开关打到 photo，插入计算机，直到计算机发现新硬件，松开录制按钮，手动安装驱动程序(光盘内有完整驱动程序)

4.关机操作： 如果需要关机，请在录像暂停状态或在拍照模式下把电源开关拨向off即可.

!! 注意：如果在录像过程中或者录音过程中关机将造成数据丢失.如果机器在此2种状态下需要转为待机状态.

5.节电模式：当机器在空闲状态时，如超过1分钟不对其进行操作，则自动关机

6.连接计算机：关机状态下把电源开关打到“ON”位置或待机状态（拍照时橙灯亮或录像时蓝灯停止闪动），可以连接计算机，此时，计算机将会弹出可移动磁盘标识.拔出DVR时，请点击右下角任务栏的可移动设备管理图示，在里面卸载该可移动存储设备，待系统确认可以安全拔出硬件后，再断开摄像笔与计算机的连接

7.充电

A.PC充电：在开机或者待机状态下，插入USB线，连接计算机USB接口，如果电池电量不足，则LED1闪动，指示现在正处于充电状态，此时请右击右下角任务栏的可移动磁盘图标，卸载该可移动存储设备后（别拔掉USB线），当电池电量充满后，蓝色灯停止闪动。

B.充电器充电: 在开机或者待机状态下，插入充电器，如果电池电量不足，如果电池电量不足，则蓝色灯闪动，指示现在正处于充电状态，当电池电量充满后，则蓝色灯停止闪动，继续充30分钟可确保完全充满.
注意：最初使用的前五次，请将电池用完后再充电至饱和，以启动电池最大容量，获取标准工作时间.

8.复位: 当产品因为非法操作或其它未明原因停止响应时（比如出现红灯长亮操作无反应时），需要对产品进行复位.把电源开关从 ON 切换至 OFF 为自动复位.

三、相关参数

1.视频档：640*480 30FPS AVI，推荐播放器：KMplayer（随机光盘附送）

2.拍照像素1600*1200  JPG

4.支持系统：windows98SE/me/2000/xp/2003/vista；linux；其中win98SE操作系统，需安装随盘附带的“win98 driver”档夹的驱动程序

5.电池类型：高容量聚合物锂电

6.充电电压：DC-5V

7.数据接口类型：Mini 5pin USB

四、修改录像时间

* 打开一个TXT格式档，里面写上当天的时间日期按图档中时间格式即可，保存档为time.txt  ，如下图:

* 拷贝time.txt文件到DVR的盘根目录里面“开机-关机”一个循环即可.

五、其它未明事宜请与当地经销商联系

六、注意事项

* 使用场合：请严格遵守国家相关法令，不得将此产品用于其它非法用途，否则后果自负

* 工作温度：请在自然温度下使用，请勿在人体不能适应的温度下工作。

* 工作湿度：请在人类适宜生活的环境湿度下使用，请勿将产品置于潮湿的工作环境，产品不具备防水功能。

* 拍摄照度：请在有充足的光源环境下使用，请勿将摄像头直接对着太阳等超强光源，以免光学器件受到损伤

* 保洁要求：请勿在粉尘密度过大的环境下使用，以免镜头以及其它部件沾染粉尘，影响摄像效果。镜头可以用擦镜纸或眼镜布轻轻擦拭，保持洁净。

* 其它事项：该产品属于精密电子产品，请勿使其受到强烈冲击、震动；请勿在强磁场、强电场下使用.